Mike's Simple Adaptive Block Compression With Silence Compression

©1994 mycal labs www.mycal.net

To Compress a Sample:

Assume BLOCK_SIZE is initialized to a multiple of 16, good values are 80 to 160.

I'm assuming that the data_in[] array is the incoming sample, and that it is filtered with the /4, /2, /4 filter.

I'm also assuming that the data_in[] array is padded to a multiple of the BLOCK_SIZE with 0x80.

Assume data_len = the length of the data_in[] array padded out to a multiple of BLOCK_SIZE;

Initialize Global State Variables before compressing a sample:

Ref_sample = 0x0;

Current_block_pointer=0;

Compress, a block at a time until there is no data left:

While(current_block_pointer<data_len)

compress a block(current_block_pointer);

current_block_pointer= current_block_pointer + BLOCK_SIZE;

Else

We are done;

Compress and store block:

Calculate Average Step Size:

Prev=data_in[current_sample_pointer] -128;			// convert sample to signed, just flip msb on

// a byte, or flip upper byte on word.

Step_size=0;

For(i=1;i<BLOCK_SIZE;i++)

{

t=(data_in[i+current_block_pointer]-128);

step_size=step_size+abs(t - prev);

prev=t;

}

step_size=step_size/(BLOCK_SIZE/2);

Store step size.

If(step_size!=0) we need to store block

Store Block:

For (i=0;i<BLOCK_SIZE;i++)

{

t=(data_in[i+current_block_pointer]-128);

if(t > ref_sample)

{

store a 1

ref_sample=ref_sample + step_size;

}

else

{

store a 0

ref_sample=ref_sample - step_size;

}

}

To Uncompress a Sample:

Assume Post Filtering with /4,/2,/4 filter.

Assume BLOCK_SIZE as above.

Assume cdata[] array is an array of bytes produced by the above routine.

Initialize Global State Variables:

Ref_sample = 0x0;

Current_block_pointer=0;

Uncompress_Block:

While(data_pointer<data_len)

Uncompress_block(data_pointer);

Else

Were done;

Get Step Size:

Step_size = cdata[data_pointer];

Data_pointer = data_pointer +1;

If(step_size == 0)

{

//replicate silence

for(i=0; i< BLOCK_SIZE;i++)

store ref_sample;

}

else

{

// replicate waveform from data

For (i=0;i<BLOCK_SIZE;i++)

{

if(bit=1)

ref_sample=ref_sample+step_size;

else

ref_sample=ref_sample-step_size;

store ref_sample;

}

data_pointer = data_pointer + (BLOCK_SIZE/8);

}

EXAMPLE - compress

This sample does not take in consideration the filter, and it can be assumed that the filter is applied before the compression takes place.

Eleven.wav - for this example we use a block size of 16.   The first 16 data bytes from eleven.wav are :

134, 133, 133, 131, 129, 127, 127, 125, 123, 120,  121,  122,  122,  125,  128,  128

2nd  16 bytes are:

122,  118,  120, 124,  132,  140,  142,  139,  131,  123,  118,  120,  127,  133,  138,  139

Converted to signed values are:

6,  5,  5, 3,  1, -1,  -1,  -3, -5,  -8, -7,  -6, -6,  -3, 0, 0

and

-6,  -10,  -8,  -4,  4, 12,  14, 11,  3, -5,  -10, -8,  -1, 5  10, 11

First block calculated step size is:

step_size = 3  	(step= 28,  28/(block_size/2), 28/(16/2), 28/8)

2nd is :

step_size = 9	(step=79, 79/8)

First Compressed Block is :

0x03 0xC8 0x97

This is Step_size =0x03  plus

Ref_sample=0, step_size=3, 1st value = 6 is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = 3

Ref_sample=3, step_size=3, 2nd value = 5 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 6

Ref_sample=6 step_size=3, 3st value = 5 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = 3

Ref_sample=3, step_size=3, 4th value = 3 is less (or =)  than Ref_Sample, Store 0

Ref_Sample=Ref_Sample - step_size = 0

Ref_sample=0, step_size=3, 5st value = 1 is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = 3

Ref_sample=3, step_size=3, 6th value = -1 is less than Ref_Sample, Store 0

Ref_Sample=Ref_Sample - step_size = 0

Ref_sample=0, step_size=3, 7th value = -1 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample + step_size = -3

Ref_sample=-3, step_size=3, 8th value = -3 is less than (or =) Ref_Sample, Store 0

Ref_Sample=Ref_Sample - step_size = -6

Ref_sample=-6, step_size=3, 9th  value = -5 is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = -3

Ref_sample=-3, step_size=3, 10th  value = -8 is less than Ref_Sample, Store 0

Ref_Sample=Ref_Sample - step_size = -6

Ref_sample=-6 step_size=3, 11th  value = -7 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = -9

Ref_sample=-9, step_size=3, 12th value = -6 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = -6

Ref_sample=-6, step_size=3, 13th  value = -6 is less than (or =)  than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = -9

Ref_sample=-9, step_size=3, 14th value = -3 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = -6

Ref_sample=-6, step_size=3, 15th value = 0 is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = -3

Ref_sample=-3, step_size=3, 16th value = 0 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 0

Second Compressed Block is :

0x09 0x3D 0x17

This is Step_size =0x09  plus

Ref_Sample is carried over from the last block, (it = 0 in this case)

Ref_sample=0, step_size=9, 17th  value = -6 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = -9

Ref_sample=-9, step_size=9, 18th  value = -10 is less than Ref_Sample, Store 0

Ref_Sample=Ref_Sample - step_size = -18

Ref_sample=-18 step_size=9, 19th value = -8  is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = -9

Ref_sample=-9, step_size=9, 20th value = -4 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 0

Ref_sample=0, step_size=9, 21st value = 4 is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = 9

Ref_sample=9, step_size=9, 22nd  value = 12 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 18

Ref_sample=18, step_size=9, 23rd  value = 14 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = 9

Ref_sample=9, step_size=9, 24th  value = 11 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 18

Ref_sample=18, step_size=9, 25th   value = 3 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample + step_size = 9

Ref_sample=9, step_size=9, 26th  value = -5 is less than Ref_Sample, Store 0

Ref_Sample=Ref_Sample - step_size = 0

Ref_sample=0 step_size=9, 27th  value = -10 is less than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = -9

Ref_sample=-9, step_size=9, 28th  value = -8 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 0

Ref_sample=0, step_size=9, 29th   value = -1 is less than  than Ref_Sample, Store 0

Ref_sample=Ref_Sample - step_size = -9

Ref_sample=-9, step_size=9, 30th  value = 5 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 0

Ref_sample=0, step_size=9, 31st value = 10 is greater than Ref_Sample, Store 1

Ref_sample=Ref_Sample + step_size = 9

Ref_sample=9, step_size=9, 32nd  value = 11 is greater than Ref_Sample, Store 1

Ref_Sample=Ref_Sample + step_size = 18